| Machine Type: | DD Mechanical Shears |
| --- | --- |
| Machine Centre No: | HPG |
| Serial No: | 91D299 |

Machine Maintenance

Manual & Log Book.

Document No: AEU00676                                                                       Revision: 01

CONTENTS.

SECTION 1 – MAINTENANCE SCHEDULE.

- Start of Production Checks.

- Weekly Checks.

- Monthly Checks.

- Six Monthly Checks.

- Two Yearly Checks.

SECTION 2 – SIGN OFF SHEETS.

- Start of Production Checks.

- Weekly Checks.

- Monthly Checks.

- Six Monthly Checks.

- Two Yearly Checks.

SECTION 3 – MACHINE DOWNTIME RECORED.

SECTION 4 – IMPORTANT CONTACTS.

SECTION 5 – CRITICAL SPARES LIST.

SECTION 6 – CHANGE NOTES.

- MAINTENANCE SCHEDULE.

- Start of Production Checks.

These checks are to be completed before production starts on every shift when the machine is used.

- Complete Machine – Brush off any dust, dirt. Etc. from the machine table. Lightly oil table top and shearing blades.

- Weekly Checks.

These checks must be carried out once a week when the machine is used.

- Connecting rod eccentric bearings (2 off) – Lubricate bearings by grease gun application.

- Hold -down slides, shearing beam slideways and ‘quick set’ device – Lubricate by oil can application.

- Standard backgauge bars and manual back-gauge – Wipe clean and grease lubricate by hand.

- Monthly Checks.

These checks must be carried out once a month when the machine is used.

- Footswitch – Examine footswitch wandering lead for general deterioration & damage. Do not allow lead to become contaminated with oil.

- Drive belts – Inspect belts for cracking, general deterioration and tension (See page 4.10 of manual for details of tension adjustment.

- Speed reducer gearbox & torque bar – Check oil level and replenish as necessary.

- Speed reducer gearbox & torque bar – Check tighten the torque bar lock-nut and fixings.

- Six Monthly Checks.

- Hold-down spring assemblies – Remove, clean and re-grease.

- Speed reducer gearbox & torque bar – Drain and flush gearbox, clean magnetic drain plug and replenish with fresh oil.

Caution: Too much oil will cause overheating, too little oil will cause gear failure.

- Drive motor and brake – Check brake ring for wear and adjust brake as necessary.

- Electrical control gear – Check condition and security of all control gear in cabinet and limit switches on machine. Worn, loose or burnt contacts and faulty items should be replaced immediately.

- Main shaft plummer block bearings – Lubricate bearings by grease gun application.

- Two Yearly Checks.

- Drive motor and brake – Renew grease in bearings.

- SIGN OFF SHEETS.

- Start of Production Checks.

| Date (dd-mmm-yy) | Name (Print) | Signature | Comments |
| --- | --- | --- | --- |

- Weekly checks (When in Production).

| Date (dd-mmm-yy) | Name (Print) | Signature | Comments |
| --- | --- | --- | --- |

- Monthly checks.

| Date (dd-mmm-yy) | Name (Print) | Signature | Comments |
| --- | --- | --- | --- |

- Six Monthly Checks.

| Date (dd-mmm-yy) | Name (Print) | Signature | Comments |
| --- | --- | --- | --- |
| - Two Yearly Checks. |  |  |  |
| Date (dd-mmm-yy) | Name (Print) | Signature | Comments |

- MACHINE DOWNTIME RECORED.

| Date | Description | Batch No. | Repair Date |
| --- | --- | --- | --- |
| Date | Description | Batch No. | Repair Date |
| Date | Description | Batch No. | Repair Date |
| Date | Description | Batch No. | Repair Date |
| Date | Description | Batch No. | Repair Date |
| Date | Description | Batch No. | Repair Date |
| Date | Description | Batch No. | Repair Date |
| Date | Description | Batch No. | Repair Date |

- IMPORTANT CONTACTS.

| Name | Company / Address | Telephone No. |
| --- | --- | --- |
| Luke Handley | Powerplus Electrical | 0115 7797786 07866 782015 |
| John Carsley | Trent oils Phoenix House Kemmel Road, Bulwell Nottingham Nottinghamshire NG6 9FH | 0845 2701122 |
| Kelvin Dennis | KD Electrical | 07702 401173 |

- CRITICAL SPARES LIST.

Hydraulic Oil 32

- CHANGE NOTES.

| Revision Level | Description | Date | Name |
| --- | --- | --- | --- |
| 01 | First Issue | 17TH Mar 2017 | Christopher Rowntree |